SLS-RESTRICTED // PROGRAM DEEPWATER // NEED-TO-KNOW

# STURGEON (SLS-201) — Sea-Trial Results Summary

| PROGRAM | DEEPWATER / STURGEON / SLS-201 |
| --- | --- |
| SERIES | Trial block 3 |
| DATE | 18 February 2026 |
| CONTROL | SLS-RESTRICTED — DEEPWATER need-to-know |

Block 3 trials advanced STURGEON against its performance set. Results below are measured unless marked modeled. Several items remain in test. Handle as DEEPWATER need-to-know; this program is not acknowledged externally.

Results

Dive to 2,500 m completed; hull and seals nominal, no ingress.

18.2 kt sprint sustained for 12 minutes; 12 kt cruise verified.

21-day unsupervised patrol completed under Level 4+ autonomy; two operator interventions logged.

Deployed and recovered ROCKFISH (SLS-207) UUVs: 4 of 6 recovered cleanly; 2 recovery aborts (dock alignment).

Acoustic signature measured ~6 dB below the ROCKFISH baseline (anechoic coating).

Endurance modeled at 96 days at patrol load; longest physical soak to date is 31 days.

INS + gravimetric nav held 38 m error over 21 days with no GNSS fix.

Open issues & next steps

Resolve swarm-recovery dock alignment before block 4 (root cause: cross-current capture).

Continue endurance soak toward the 90-day target; AIP fuel-cell stack is the long pole.

Block 4: extend unsupervised patrol to 45 days; integrate the modular EW payload.

Brief the Board sub-committee under DEEPWATER access only.